Control pilot:

# Step 1: Inside the Charger

Forget about PWM for a moment.

Imagine the charger has a simple +12 V DC source.

Inside the charger:

+12V

             │

           1 kΩ

             │

             ●────────────── CP Pin

             │

             │

           (Nothing)

             │

            GND

Notice something.

The CP pin is not directly connected to +12 V.

There is always a 1 kΩ resistor in series.

That resistor is extremely important.

# Why is there a 1 kΩ resistor?

Without it:

+12V

 │

 ├──────── CP

Suppose someone accidentally shorts CP to PE.

Current would be:

I=VRI = \frac{V}{R}I=RV​

For a short:

12V / 0Ω

=

∞

Very large current.

Something burns.

Instead:

12V

↓

1kΩ

↓

CP

Maximum current:

12V / 1000Ω

=

12 mA

Now the circuit is protected.

The resistor also forms the basis of the voltage divider used for state detection.

# Step 2: No Vehicle Connected (State A)

Nothing is connected.

+12V

 │

1kΩ

 │

●──── CP

 │

Open Circuit

Since no current flows:

Current = 0 A

Voltage drop across the 1 kΩ resistor:

V = I × R

0 × 1000

=

0V

So:

Top of resistor:

12 V

Bottom of resistor (CP):

12 V

Therefore:

CP = +12 V

The charger reads:

Vehicle absent.

# Step 3: Vehicle Plugged In

Now comes the clever part.

Inside every EV there is a resistor connected from CP to Protective Earth (PE).

Initially:

2.74 kΩ

Charger

+12V

 │

1kΩ

 │

CP───────────────┐

                 │

               2.74kΩ

                 │

                 │

                PE

Now current flows.

Let's calculate it.

Total resistance:

1000 + 2740

=

3740 Ω

Current:

12 / 3740

=

3.21 mA

Now calculate the voltage across the 1 kΩ resistor.

3.21mA × 1000

=

3.21V

The resistor "uses up" 3.21 V.

The remaining voltage is:

12

-

3.21

=

8.79V

Rounded:

≈ 9 V

Now you understand where +9 V comes from.

The vehicle never generated 9 V.

It simply connected a resistor.

The charger generated the voltage drop.

# This is Just a Voltage Divider

The CP circuit is fundamentally:

1kΩ

12V ─────/\/\/──────●───── ADC

                    │

                 2.74kΩ

                    │

                   PE

The CP voltage is simply the divider output.

# Step 4: Vehicle Ready to Charge

The EV now closes an internal switch.

Another resistor is added.

Equivalent resistance becomes approximately:

882 Ω

Charger

12V

 │

1kΩ

 │

CP─────────────┐

               │

             882Ω

               │

               │

              PE

Let's calculate.

Total resistance:

1000 + 882

=

1882 Ω

Current:

12 / 1882

=

6.38 mA

Voltage across 1 kΩ:

6.38V

Remaining voltage:

12

-

6.38

=

5.62V

Approximately:

6 V

Again—

The EV never generated 6 V.

It only changed the resistor value.

# Why Those Specific Resistor Values?

Now you're asking the right engineering question.

Why not:

- 2 kΩ?

- 3 kΩ?

- 5 kΩ?

The values defined by the charging standard are chosen to create well-separated voltage windows.

Example:

| Vehicle Resistance | CP Voltage |
| --- | --- |
| Open | 12 V |
| 2.74 kΩ | ~9 V |
| 882 Ω | ~6 V |
| 246 Ω | ~3 V |

These voltages are far enough apart that tolerances in resistors, temperature changes, and ADC errors are unlikely to cause misclassification.

# But the STM32 Can't Read 12 V!

Correct.

STM32 ADC:

0–3.3V

CP:

±12V

The MCU never connects directly to the CP line.

Instead, the charger has a CP interface circuit.

A simplified version looks like:

CP

↓

Protection Diodes

↓

Voltage Divider

↓

RC Filter

↓

Comparator / Op-Amp

↓

ADC

Let's examine each stage.

# Stage 1: Voltage Divider

Suppose we choose:

100 kΩ

and

33 kΩ

CP

 │

100kΩ

 │

────── ADC

 │

33kΩ

 │

GND

If CP = 12 V:

ADC input becomes approximately:

3 V

Safe for the MCU.

# Stage 2: RC Filter

Divider

↓

10kΩ

↓

100nF

↓

ADC

Purpose:

- Remove noise

- Smooth ringing

- Reduce EMI

- Improve ADC stability

# Stage 3: Protection

Usually:

CP

↓

Series Resistor

↓

TVS

↓

Schottky Diodes

↓

ADC

These protect the MCU from transients and overvoltage.

# How Does the MCU Read the Voltage?

The STM32 periodically samples the conditioned CP voltage.

Example:

Actual CP:

9 V

Divider ratio:

4:1

ADC sees:

2.25 V

Firmware converts the ADC value back to the original CP voltage.

Example logic:

if(cp_voltage > 10.5)
    STATE_A;

else if(cp_voltage > 7.5)
    STATE_B;

else if(cp_voltage > 4.5)
    STATE_C;

else if(cp_voltage > 2)
    STATE_D;

else
    FAULT;

This is a simplified illustration; production code uses thresholds with tolerances and filtering.

# But Wait... What About the Negative Half Cycle?

Now we're reaching the really interesting part.

So far we've only discussed the positive level.

However, the actual Control Pilot signal isn't just +12 V.

It alternates between approximately:

+12V

↓

-12V

↓

+12V

↓

-12V

at 1 kHz.

The negative half-cycle serves several purposes:

- Verifies the diode inside the vehicle.

- Detects wiring faults.

- Helps distinguish valid vehicle circuits from certain fault conditions.

- Ensures compliance with the charging standard.

The vehicle includes a diode in series with its resistor network, so the charger sees different behavior during the positive and negative halves of the waveform. This is how it can verify that a compliant vehicle is connected rather than just a resistor.

# First, What Does the Charger Actually Generate?

The charger generates a 1 kHz square wave that swings between approximately +12 V and -12 V with respect to Protective Earth (PE).

For a 50% duty cycle:

+12V ───────┐      ┌───────
            │      │
            │      │
0V ─────────┼──────┼────────
            │      │
            │      │
-12V        └──────┘

One cycle = 1 ms

- Positive portion = Duty Cycle

- Negative portion = Remaining time

Example:

50% duty

+12V → 500 µs

-12V → 500 µs

# Why Not Just Generate 0–12 V PWM?

This is the first important question.

Suppose the charger generated:

12V

0V

12V

0V

The problem is:

The charger could not verify whether:

- A real EV is connected

- The cable is damaged

- Someone has connected random resistors

- The CP wire is broken

- The vehicle electronics are healthy

There would be much less information available from the signal.

# The Vehicle Contains a Diode

This is the key component.

Inside every compliant EV:

CP

 │

>|

 │

Resistors

 │

PE

Notice:

The diode is in series with the resistor network.

This diode is required by the charging standard.

# Why Is the Diode There?

A diode allows current in only one direction.

When CP is positive:

+12V

↓

Diode

↓

Resistor

↓

PE

The diode conducts.

Current flows.

The charger measures the expected CP voltage (for example, about +9 V or +6 V depending on the state).

# What Happens During the Negative Half-Cycle?

Now the charger drives CP to approximately -12 V.

-12V

↓

Diode

The diode is now reverse-biased.

It blocks current.

No current should flow through the resistor network.

This gives the charger additional information.

# Let's Compare

## Positive Half-Cycle

CP = +12V

↓

Diode ON

↓

Resistor Connected

↓

Voltage Drops

↓

Charger Measures

## Negative Half-Cycle

CP = -12V

↓

Diode OFF

↓

Resistor Disconnected

↓

Different Voltage Behaviour

↓

Charger Measures

The charger is effectively checking:

"Does this circuit behave like a real EV input?"

# Why Is This Useful?

Imagine there were no diode.

Someone accidentally connected:

CP

↓

2.74kΩ

↓

PE

The charger would measure:

Approximately +9 V

Everything appears normal.

But this isn't necessarily a valid EV interface.

Now look at the negative cycle.

## Without Diode

CP = -12V

↓

Resistor

↓

PE

Current flows in the reverse direction.

The charger immediately notices:

"This doesn't look like a compliant vehicle."

## With Diode

CP = -12V

↓

Reverse-biased diode

↓

No current

Exactly what the standard expects.

# The Negative Cycle is a Self-Test

Think of it as a built-in diagnostic.

Every millisecond the charger asks:

Positive cycle:

"Which resistor do I see?"

Negative cycle:

"Is the diode really there?"

Only if both answers are correct does the charger trust the connection.

# What Voltage Does the Charger Actually Measure?

Here's the interesting part.

Many people think:

Positive:

+9V

Negative:

-9V

This is not what happens.

During the negative half-cycle:

The diode blocks current.

Therefore the CP line behaves differently than it does during the positive half-cycle.

Depending on the exact implementation and sensing circuitry, the charger expects to see the characteristic negative level associated with a compliant diode/resistor network. If that expected behavior is missing, it indicates a fault or non-compliant connection.

# What Faults Can the Negative Cycle Detect?

It helps detect situations such as:

### 1. Missing Diode

Vehicle electronics damaged

↓

Diode shorted or absent

↓

Negative current flows incorrectly

↓

Fault detected

### 2. Wrong Cable

Random resistor connected

↓

No diode

↓

Rejected

### 3. Short Circuit

CP

↓

PE

Immediately detected.

### 4. Wiring Errors

Incorrect connections on the CP line can produce an unexpected response during the negative portion of the waveform, allowing the charger to identify faults before energizing the power contacts.

# Why Doesn't the Vehicle Generate the PWM?

Another common misconception.

Many engineers initially think:

Vehicle generates response PWM.

No.

The vehicle is almost entirely passive during the basic CP handshake defined by the charging standard.

The charger continuously generates:

+12V

↓

-12V

↓

+12V

↓

-12V

The vehicle simply changes:

- Resistor values

- Switch positions

- Diode conduction path

Everything else is inferred by the charger.