SENSORS:

List:

- Current sensor

- Voltage sensor

# Overall Voltage Sensing Block

230 VAC / 415 VAC
                  │
                  ▼
        Input Protection Circuit
                  │
                  ▼
         Voltage Sensing Circuit
                  │
                  ▼
      Isolation (if required)
                  │
                  ▼
        Signal Conditioning
                  │
                  ▼
              ADC Input
                  │
                  ▼
             STM32 MCU
                  │
                  ▼
         RMS & Fault Detection

Voltage Scaling:

# Method 1 – Resistor Divider (Most Common)

230 VAC
   │
  R1
   │──────────────► ADC
  R2
   │
 Neutral

The resistor divider reduces the voltage according to the divider ratio.

The output voltage is:

Vout=Vin×R2R1+R2V_{out}=V_{in}\times\frac{R_2}{R_1+R_2}Vout​=Vin​×R1​+R2​R2​​

For example:

- R1 = 1 MΩ

- R2 = 10 kΩ

This scales the voltage by roughly 100:1.

### Advantages

- Simple

- Low cost

- Accurate

- Easy to design

### Disadvantages

- No electrical isolation

- Requires careful PCB layout

- High-voltage safety must be considered

# Method 2 – Potential Transformer (PT)

230 VAC

↓

Potential Transformer

↓

6 VAC

↓

Rectifier

↓

Filter

↓

ADC

### Advantages

- Excellent electrical isolation

- Very safe

- High noise immunity

### Disadvantages

- Larger size

- Higher cost

- Lower bandwidth

- Less common in compact AC chargers

Reasons to use PT over regular transformer:

# 1. High-Quality Core Material

A regular transformer may use standard silicon steel laminations.

A PT uses:

- High-grade grain-oriented silicon steel

- Low-loss magnetic materials

- Carefully laminated cores

This reduces:

- Hysteresis loss

- Eddy current loss

- Magnetizing current

Result:

- More accurate voltage transformation.

# 2. Very Precise Turns Ratio

Suppose the design requires exactly:

Primary Turns = 10,000
Secondary Turns = 100

Ratio = 100 : 1

A power transformer might tolerate small manufacturing variations.

A PT is wound with much tighter tolerances so that the output remains very close to the intended ratio.

Example:

11000 V

↓

109.98 V

instead of

11000 V

↓

108 V

# 3. Operates at a Very Light Load (High Burden Accuracy)

A power transformer is designed to supply significant current.

Example:

230V
 │
Transformer
 │
10 A Load

As load current changes, voltage drops occur due to the winding resistance and leakage reactance.

A PT normally feeds only:

- Voltmeters

- Protective relays

- PLC analog inputs

- Energy meters

These devices draw very little current.

11kV
 │
PT
 │
Voltmeter

Because the load is tiny, internal voltage drops are minimal, so the output stays accurate.

# 4. Better Winding Arrangement

PT windings are arranged carefully to reduce leakage flux.

Simplified view:

Primary
██████

Secondary
██████

Better magnetic coupling means:

- Lower leakage inductance

- Smaller voltage error

- Lower phase shift

# 5. Reduced Phase Error

Besides voltage accuracy, a PT also preserves the phase relationship.

Imagine:

Actual input

sin(ωt)

Poor transformer

sin(ωt + 5°)

PT

sin(ωt + 0.1°)

Small phase error is essential for:

- Power factor calculations

- Protective relays

- Energy metering

# 6. Accuracy Classes

PTs are manufactured to specific accuracy standards.

Typical classes include:

| Accuracy Class | Maximum Ratio Error |
| --- | --- |
| 0.1 | ±0.1% |
| 0.2 | ±0.2% |
| 0.5 | ±0.5% |
| 1.0 | ±1.0% |

A general-purpose transformer usually isn't specified this way because precise measurement isn't its job.

# 7. Low Burden Rating

PTs are rated in VA (Volt-Amperes), but the values are intentionally small.

Examples:

- 10 VA

- 15 VA

- 25 VA

- 50 VA

This is enough to power measuring instruments but not large loads.

# 8. Excellent Voltage Regulation

When the load changes:

### Power Transformer

No Load
12.2V

Full Load
11.3V

The output voltage changes noticeably.

### Potential Transformer

No Load
110.0V

Rated Burden
109.9V

Very little change.

# 9. Better Insulation

Since PTs often connect to medium- and high-voltage systems, they use insulation designed to withstand high electrical stress for long periods while maintaining measurement accuracy.

# Method 4 – Voltage Sensor IC

Some designs use dedicated measurement ICs.

Example architecture:

AC Line

↓

Divider

↓

Energy Meter IC

↓

SPI / UART

↓

STM32

In this case, the MCU doesn't directly sample the voltage. The measurement IC computes quantities such as RMS voltage and communicates digitally.

# Signal Conditioning

The scaled signal often passes through conditioning circuitry before reaching the ADC.

Typical stages include:

Divider

↓

RC Low-Pass Filter

↓

Buffer Op-Amp

↓

ADC

### Why?

- Reduce noise

- Protect the ADC

- Improve measurement accuracy

- Match the ADC input impedance

# Types of RC Filters

## 1. Low-Pass Filter (LPF)

Passes low frequencies and attenuates high frequencies.

Vin
 │
 R
 │────── Vout
 │
 C
 │
GND

### Frequency Response

Gain

│───────────────
│              \
│               \
│                \
│_________________\________ Frequency
          fc

Applications:

- ADC input filtering

- Sensor noise removal

- Voltage sensing

## 2. High-Pass Filter (HPF)

Passes high frequencies and blocks low frequencies.

Vin
 │
 C
 │────── Vout
 │
 R
 │
GND

Applications:

- Remove DC offset

- Audio coupling

- Signal conditioning

# Cutoff Frequency

The most important parameter is the cutoff frequency fcf_cfc​:

fc=12πRCf_c=\frac{1}{2\pi RC}fc​=2πRC1​

At this frequency:

- Output amplitude is about 70.7% of the input.

- This corresponds to −3 dB.

# Design Procedure

## Step 1: Decide What You Want to Filter

Example:

Sensor bandwidth:

- Useful signal: 0–50 Hz

- Noise: above 500 Hz

Choose:

Cutoff ≈ 100 Hz

## Step 2: Choose R or C

Usually one component is chosen based on practical considerations.

Example:

R = 10 kΩ

## Step 3: Calculate the Other Component

Given:

fc = 100 Hz
R = 10 kΩ

Use:

C=12πRfcC=\frac{1}{2\pi Rf_c}C=2πRfc​1​

Substitute:

C=12π(10000)(100)C=\frac{1}{2\pi(10000)(100)}C=2π(10000)(100)1​ C=1.59×10−7FC=1.59\times10^{-7}FC=1.59×10−7F

≈ 160 nF

Choose the nearest standard value:

150 nF

or

160 nF

# Measuring an AC Waveform

The mains voltage is sinusoidal.

+325 V
         /\
        /  \
-------/----\-------
      /      \
     /        \
   -325 V

An MCU ADC cannot measure negative voltages directly.

There are two common approaches.

## Option 1 – DC Bias

A DC offset is added so the entire waveform sits within the ADC range.

3.3 V

      /\

2.5 V ---- Center Bias

      \/

0 V

This allows the ADC to sample the complete waveform.

## Option 2 – Rectification

The AC waveform is rectified before measurement.

AC

↓

Bridge Rectifier

↓

Capacitor

↓

DC Voltage

↓

ADC

Current Sensing in EV Chargers

# B) Current Sensing Block Diagram

AC Input
           │
           ▼
      Main Contactor
           │
           ▼
     Current Sensor
           │
           ▼
 Signal Conditioning
           │
           ▼
        ADC / Meter IC
           │
           ▼
         STM32 MCU
           │
           ▼
 Protection + Billing + Control

# Where is the Sensor Placed?

For a single-phase charger:

Live ───── Current Sensor ───── Vehicle

Neutral ─────────────────────── Vehicle

Usually only the Live conductor is measured.

For a three-phase charger:

L1 ── Sensor 1 ──►

L2 ── Sensor 2 ──► Vehicle

L3 ── Sensor 3 ──►

Each phase normally has its own sensor.

# 1. Shunt Resistor

This is the simplest current sensing method.

## Principle

A small resistor is placed in series with the load.

AC Line

───────Rshunt────────Vehicle

As current flows, a small voltage appears across the resistor.

The MCU measures that voltage.

# 2. Hall-Effect Current Sensor

This is probably the most common sensor you'll encounter in commercial AC EV chargers.

## Principle

It measures the magnetic field created by current flowing through a conductor.

No electrical contact is required.

Wire

↓

Magnetic Field

↓

Hall Sensor

↓

Voltage Output

# 3. Current Transformer (CT)

Very common in industrial AC systems.

## Principle

A CT works like a transformer.

Primary Wire

↓

Magnetic Core

↓

Secondary Coil

↓

Measurement Resistor