Power terminologies :

# Peak Voltage vs RMS Voltage

Look at a normal AC sine wave.

+325V
             ▲
             │
             │
      /\     │
     /  \    │
----/----\---┼---------
   /      \
  /        \
             │
             │
             ▼
          -325V

The highest value is called the Peak Voltage.

Vpeak = 325V

But when we say

230 VAC

we are not talking about the peak.

We are talking about the RMS voltage.

# Relationship Between Peak and RMS

For a pure sine wave:

VRMS=VPeak2V_{RMS}=\frac{V_{Peak}}{\sqrt2}VRMS​=2​VPeak​​

Therefore:

325 / 1.414

=

230V

Similarly,

230 × 1.414

=

325V

So a 230 V mains supply actually swings between approximately +325 V and -325 V.

# Three Types of Power

There are three kinds.

Total Power
               │
     ┌─────────┴─────────┐
     │                   │
Useful Power      Reactive Power

Engineers call them:

- Real Power

- Reactive Power

- Apparent Power

# 1. Real Power (P)

Real Power is

The power that actually performs useful work.

Examples

Bulb

↓

Light

Heater

↓

Heat

Motor

↓

Mechanical Rotation

EV Charger

↓

Charges Battery

Unit

Watt (W)

# 2. Reactive Power (Q)

Reactive power does no permanent work.

Instead,

energy is stored

then returned

many times every second.

Think of it like this.

Imagine a swing.

Push →

Swing

← Returns

Energy keeps moving back and forth.

Nothing is permanently consumed.

Inductors do this.

Source

↓

Magnetic Field

↓

Source

Capacitors do the same.

Source

↓

Electric Field

↓

Source

The energy oscillates between the source and the reactive component.

Unit

VAR

Volt-Ampere Reactive

# 3. Apparent Power (S)

This is

The total power the source must supply.

Imagine a company.

Employees:

10

Only

8

are doing useful work.

The other

2

are occupied with moving materials around.

The company still pays all

10

Similarly,

the grid must provide

both

- useful power

- reactive power

Their combination is called Apparent Power.

Unit

VA

Volt-Ampere

# Why Does an EV Charger Care?

Suppose your charger draws

230V

10A

The apparent power is

2300VA

But if

Power Factor

=

0.90

Then

Real Power

2300 × 0.9

=

2070W

Only

2070W

actually charges the battery.

The rest is associated with reactive power.

# Pure Resistive Load

Example

Heater

Voltage and current are perfectly aligned.

Voltage

/\/\/\/\/\

Current

/\/\/\/\/\

Real Power

100%

Reactive

0%

Power Factor

1

# Pure Inductor

Voltage

/\/\/\/\/\

Current is delayed.

Current

   /\/\/\/\/\

Some energy is stored in the magnetic field and then returned to the source.

Real power is low compared with apparent power.

# Pure Capacitor

Current leads voltage.

Again,

energy moves back and forth.

# Why This Matters in EV Chargers

Modern EV chargers don't simply calculate:

Power = Voltage × Current

They also determine the phase relationship between voltage and current. That allows them to compute:

- RMS Voltage

- RMS Current

- Real Power (W)

- Apparent Power (VA)

- Reactive Power (VAR)

- Power Factor (PF)

- Energy (kWh)

These values are important for efficiency, billing, grid compliance, and diagnostics.